Severity Rankings for Tool Repairs

Answer the question:  Why does this tool need to be repaired?

This tool needs to be repaired because:

1:  It’s going to shut down a line at the customer, require expedited shipping, exhaust days on hand, or affect personal safety in any way.  Tool is not runnable.

Examples:  Need parts or Ford shuts a line down; hot oil line is broke and could burn an operator

2:  We can’t get a passing part, even with a secondary operation; running the tool will cause damage to the machine or further damage to the tool.  Tool is not runnable.

Examples:  Flashed mold, cracked slide or pin, parts are undersize and there is no way to get them into spec

3:  Making parts will require a secondary operation and/or the shipment window will be decreased.

Examples:  High flash that needs to be trimmed, gates too high needing to be trimmed, using someone to perform an extra task that normally is not needed

4:  It is displaying progressive wear that has the potential to make a part that needs a secondary operation.  There are no problems to make shipment.

Examples:  Part has flash but it’s still to print, tool wear is causing a dimension to move towards an upper or lower limit.  The job is an OP job where the OP can take care of the issue.

5:  Even though it makes good parts, it causes an inconvenience.  There are no problems to make shipment.

Examples:  Parts have a burn that requires the mold to be wiped down; the process needs to be monitored

TL-1:  CUST. /SAFETY

TL-2: DAMAGE

TL-3: 2ND OPERATION

TL-4: PROGRESSIVE

TL-5: INCONVENIENCE